Radiomics as a data mining from ultrasound images to differentiate various objects

Davood Khezerloo* 1 , Yunus Soleymani 2

1* Department of Radiology, Faculty of Alliance  Medical Sciences, Tabriz University of Medical Sciences Tabriz

2 Department of Neuroimaging, School of Advanced Medical Technologies, Tehran University of Medical Sciences, Tehran, Iran

khezerlood@tbzmed.ac.ir

Abstract— Background : The radiomics features may be affected by ultrasound imaging scan parameters. Therefore, the aim of this study is to figure out the robust and stable radiomics features to differentiate various echogenic objects in different ultrasound scan settings and parameters. Material and method: Ultrasound (US) images were carried out from GAMMEX phantom in eleven random scan setting and parameters. The regions of interest (ROIs) were manually delineated around hyperechoic, hypoechoic, isoechoic (background) objects of the GAMMEX phantom. 40 textural radiomics features were extracted from each ROI. The discrimination performance of the features across three echogenic objects (hyper- , hypo- , and isoechoic objects) was assessed by the nonparametric Mann-Whitney U test with P<0.05. Results: Even in different ultrasound scan parameters, 10 of 40 radiomics features significantly differentiated hypoechoic from the isoechoic objects, and 15 of 40 radiomics features significantly differentiated for hyperechoic from the isoechoic objects. Conclusion: In summary, we concluded that in ultrasound imaging, even in different scan settings and parameters, several robust textural radiomics features may be found that are stable in various scan settings, as well as can differentiate various echogenic objects.

Keywords: Ultrasound imaging, sonography, medical image, Radiomics, Quantitative analysis.

رادیومیکس به عنوان یک داده کاوی عددی از تصاویر پزشکی اولتراسوند برای تمایز اجسام مختلف

داود خضرلو1*،  یونس سلیمانی2

1*  گروه رادیولوژی دانشکده پیراپزشکی ، دانشگاه علوم پزشکی تبریز، تبریز ، ایران

2  گروه تصویربرداری عصبی دانشکده علوم نوین پزشکی ، دانشگاه علوم پزشکی تهران، تهران ، ایران

khezerlood@tbzmed.ac.ir

چکیده:

مقدمه: فیچرهای(ویژگیهای) رادیومیکس ممکن است تحت تأثیر پارامترهای اسکن تصویربرداری اولتراسوند قرار بگیرند. بنابراین، هدف از این مطالعه استخراج فیچرهای رادیومیکس قوی و پایدار برای تمایز اجسام اکوژنیک مختلف در تنظیمات و پارامترهای مختلف اسکن تصویربرداری اولتراسوند است. مواد و روش: تصاویر اولتراسوند  از فانتوم کنترل کیفی GAMMEX در یازده شرایط اسکن مختلف انجام شد. حدود نواحی موردنظر برای آنالیز (ROI)  به صورت دستی و به شکا دایره درون تصویر اجسام  هایپراکو، هایپواکو و ایزواکو(پس‌زمینه) فانتوم GAMMEX ترسیم شدند. 40 ویژگی رادیومیکس textural از هر ROI استخراج شد. عملکرد فیچرها برای تمایز اجسام هایپواکو و هایپراکو از ایزواکو با آزمون ناپارامتری من ویتنی U با P<0.05 ارزیابی شد. نتایج: حتی در شرایط اسکن مختلف تصویربرداری اولتراسوند، 10 مورد از 40 ویژگی رادیومیکس به طور قابل توجهی توانست اجسام هایپواکو را از اجسام ایزواکو متمایز کنند، و 15 مورد از 40 ویژگی رادیومیکس توانست به طور قابل توجهی اجسام هایپراکو را از اجسام ایزواکو متمایز کنند. نتیجه‌گیری: به طور خلاصه، به این نتیجه رسیدیم که در تصویربرداری اولتراسوند، حتی در تنظیمات و پارامترهای مختلف اسکن، چندین فیچر رادیومیکس textural قوی و پایدار میتوان یافت که توانایی تمایز اجسام با اکوژنیتی مختلف را م داشته باشند.

کلید واژگان: تصویربرداری اولتراسوند، سونوگرافی ، ، تصویر پزشکی رادیومیکس، آنالیز کمّی.

Background:

Radiomics in which a large number of quantitative features are extracted from medical images has become an attractive arena of medical researches [1]. It is a data mining approach in image pixels to help personalized-medicine and improve decision-making. With a quantitative-based analysis of heterogeneity of lesion, radiomics could overcome the subjectivity visual interpretation challenges in radiology [2]. Generally, it is assumed that medical images have more information than can be appreciated by the naked eye. The fundamental hypothesis of radiomics is based on the fact that heterogeneity inside medical images (that hardly recognized with the naked eye) may have been due to molecular phenotype, and/or genotype [3].Radiomics can be used in prediction, prognosis, classifying tumors (benign or malignant), determining mutation status, and monitoring treatment response. In the last decade, it has been used to drawing associations between clinical outcomes and quantitative data derived from radiomics by various clinical imaging techniques such as computed tomography (CT), magnetic resonance imaging (MRI), positron emission tomography (PET), and ultrasound[4-8].Quantitative features derived from ultrasound images have been demonstrated to be useful in distinguishing between normal, malignant, and benign tissues[9]. It is extensively used in breast, thyroid, and liver lesions [10-16]. In previous studies, ultrasound radiomics was often been used diagnosis, prognosis and response to treatment. The predictive and prognostic power of radiomics-based modeling have been reported moderate (0.65 < AUC < 0.8), while, when the radiomics results are accompanied by nomogram data, about 10% is added to the prediction results (0.75 < AUC < 0.9)[12, 13, 17].

Despite the advantages of radiomics, variation of scan settings and parameters such as frequencies, depth, probe, TCG,… may be affected the robustness reproducibility of its results. So, this fundamental reliability-based challenges can hind all attempts to the usage of this quantitative method the make accurate decision-making in clinics. In MRI, CT, and PET scans, radiomics features variation in the different settings have been directly studied only in some lesions. Indeed, previous researches have indicated that radiomics features are strongly sensitive to the variation of the noise level of images; therefore, any parameter that affects the noise of the image such as slice thickness, spatial resolution, and image processing kernels should be considered [18-21].

Scan parameters are highly vulnerable and variable during the examination, this may sometimes be beyond the control of the technician or specialist. ​In this phantom-base study, In this study, we intend to seek if it is possible to extract robust and stable radiomics features that are not dependent on scan parameters, as well as they have the ability to discriminate different echogenic objects. In other words, we want to seek out several common features in different US scan settings with discriminating power.

Materials and Methods:

In this phantom base study, a GAMMEX Ultrasound 403 Phantom (Sun Nuclear Corporation, NeuIsenburg, Germany) was used. Ultrasound images were also obtained by using the SAMSUNG UGEO WS80A, an Ultrasound unit (SAMSUNG MADISON CO., LTD. Seoul, Korea).  Eleven scan setting with random frequencies, various random depth and focus were used to carry out US images. Scan parameters are summarizes in table (1).

Table 1. Different but arbitrary ultrasound scan settings and parameters for Samsung unit

| Study description | Renal- Pediatric Hip- Adnexa-adnexa- abdomen- Aorta- Genecology |
| --- | --- |
| Transducer | L3-12A (linear) CA1-7A (Convex) |
| Frequencies | -Low Frequency (Penetration) -Normal Frequency (General) -High Frequency (Resolution) |
| Scanning depth | 15 different scanning depths in the range of 3.5cm - 20 cm |
| gain | 43,50, 63,71, 83, 100 |
| dynamic range | 100, 105, 106, 113, 114,116, 117 ,126 dB |
| Frame rates | 16, 18, 20, 24, 28, 29, 30, 35, 36, 41, 43 Hz |
| TGC | Random switching |
| focus | Random switching |
| zoom | Random switching |
| Thermal index | - - 0.2 - 0.3 - 0.5 |
| Mechanical Index | 0.7 - 0.8 - 0.94 - 1.3 - 1.4 - 1.5 - 2.4 |

Figure 1. components of Gammex ultrasound phantom

The phantom is composed of several standard cylindrical objects with different diameters, different acoustic impedance embedded at various depths of a soft tissue-like material. Acoustic attenuation coefficient of context is about 0.5-0.7 dB / cm / MHz and its ultrasonic speed is about 1540 m / s. ​The 10 mm diameter circular objects with acoustic impedance as Anechoic, Hypoechoic (-6 dB), and Hyperechoic (+6 dB) and a high scatter. Each object produces a different signal intensity (grayscale). There are also another objects inside the phantom to assessment axial and lateral resolution, Horizontal and vertical distance, but they were not used in this study.

ROI Delineation and Radiomics Feature Analysis: The goal of this study was the extraction of robust to distinguishing various echogenic tissue (tumoral tissue vs. normal tissue). For this purpose, several ROI delineation were done by a radiologist with more than 10 years of experience in US imaging.  For each image three ROIs were delineated manually inside the images of hypoechoic, hyperechoic, and isoechoic objects on each US image using 3D Slicer software version 4.10.2 (Fig.1). Textural features measure the spatial organization of the voxel intensities and intra-lesion heterogeneity. In this study textural features were used, because textural features are the most important radiomics features in quantitative analysis of sonography images. A total of 40 most used in recent studies textural features were extracted using open-source radiomics software, Py-Radiomics (https:// www.radiomics.io/pyradiomics.html). The features were grouped into gray-level co-occurrence matrix (GLCM, 24 features) and gray-level run-length matrix (GLRLM, 16 features). In this study, the GLCM and GLRLM features were measured using a 2D histogram with 64 bins. A total of 8 matrices corresponding to eight 2D directions with an offset of 1 were calculated and then averaged to produce a single matrix. The averaged matrix was used to compute the GLCM and GLRLM features.

All statistical analyzes were performed using MedCalc software (version 19.6.4). The nonparametric Mann-Whitney U test was used to determine the features to differentiate the hypoechoic from the isoechoic object, and then hyperechoic from the isoechoic object. It was assumed that images of hyperechoic and hypoechoic objects lesions and isoechoic objects as normal tissue. ​The significance level was set as α=0.05, so P<0.05 was considered to indicate a statistically significant difference.

Results

Figure 1 shows an ultrasound image of a GAMMEX phantom and ROI delineation in hypoechoic, hyperechoic, and isoechoic objects. In order to assess performance radiomics features for discriminating different echogenic objects, Mann-Whitney U test showed that 10 of 40 radiomics features showed significantly differentiated hypoechoic objects from the isoechoic ones.  In which, cluster shade of GLCM was the most significant radiomics feature with a P-value< 0.001.   Table 2 shows the mean, standard deviation, and p-values of the 10 significant features. According to the Mann-Whitney U test results, 15 of 40 textural features showed significantly differentiated hyperchoic objects from the isoechoic ones. Cluster shade, correlation, maximal correlation coefficient (MCC), and information measure of correlation (IMC) 2 of GLCM were the most significant features with p-values< 0.001. The mean, standard deviation, and p-values of the 15 significant features are shown in table 3.

Figure 2.  An example of ROI delineation for isoechoic, hypoechoic, and background objects on a GAMMEX phantom US image. 1) isoechoic object , 2) hypoechoic object, 3) hyperechoic object

Table 2. Robust textural radiomics features with the ability of differentiation of hypoechoic from isoechoic objects.

| Feature parameters | Mean ±SD |  | P-value |
| --- | --- | --- | --- |
|  | isoechoic | hypoechoic |  |
| Cluster shade | 0.77±0.49 | -0.008±0.30 | < 0.001 |
| Idmn | 0.99±0.00 | 0.98±0.0053 | 0.023 |
| Correlation | 0.73±0.07 | 0.61±0.12 | 0.028 |
| MCC | 0.74±0.07 | 0.62±0.123 | 0.019 |
| Sum squares | 0.52±0.17 | 0.36±0.123 | 0.028 |
| Cluster prominence | 10.31±7.14 | 4.20±3.73 | 0.010 |
| Imc 2 | 0.80±0.07 | 0.67±0.13 | 0.028 |
| Cluster tendency | 1.82±0.64 | 1.18±0.48 | 0.019 |
| Gray level variance | 0.68±0.15 | 0.47±0.15 | 0.010 |
| Gray level non-uniformity normalized | 0.34±0.037 | 0.40±0.05 | 0.023 |

Table 3. Robust textural radiomics features with the ability of differentiation of hyperechoic from isoechoic objects.

| Feature parameters | Mean ±SD |  | P-value |
| --- | --- | --- | --- |
|  | isoechoic | hyperechoic |  |
| Joint entropy | 2.48±0.35 | 2.02±0.30 | 0.002 |
| Cluster shade | 0.77±0.49 | -0.09±0.10 | < 0.001 |
| Idmn | 0.99±0.004 | 0.98±0.006 | 0.019 |
| Joint energy | 0.26±0.01 | 0.368±0.10 | 0.023 |
| Correlation | 0.73±0.07 | 0.58±0.09 | < 0.001 |
| Sum entropy | 2.20±0.30 | 1.768±0.26 | 0.001 |
| MCC | 0.74±0.07 | 0.598±0.09 | < 0.001 |
| Sum squares | 0.52±0.17 | 0.31±0.08 | 0.001 |
| Cluster prominence | 10.31±7.14 | 3.14±1.68 | 0.001 |
| Imc 2 | 0.80±0.07 | 0.64±0.10 | < 0.001 |
| Cluster tendency | 1.82±0.644 | 0.98±0.31 | 0.001 |
| Gray level variance | 0.68±0.154 | 0.49±0.08 | 0.001 |
| Gray level non-uniformity normalized | 0.34±0.04 | 0.39±0.03 | 0.001 |
| Run length non-uniformity | 172.93±94.07 | 70.90±35.76 | 0.001 |
| Run entropy | 4.64±0.24 | 4.26±0.47 | 0.016 |

Discussion

Radiomics defines the use of radiological information in a quantitative means to create relationships between imaging biomarkers and clinical outcomes to expand disease diagnosis and treatment[22]. In the previous studies, quantitative image markers were calculated based on radiomics conception that uses the US to predict prognosis or response of the disease to treatment. In our study, 15 textural features distinguished hyperechoic objects from isoechoic objects, and 10 of them also could distinguish hypoechoic objects from isoechoic objects in phantom. Despite robust results of the previous studies, radiomics features are sensitive to scan setting and parameters. Therefore, the robustness and stability of radiomics features must be recognized to provide reliable decision support for patient care. In the present study, we evaluated whether textural radiomics features are reproducible and significant across different US scan settings and parameters. The nonparametric Mann-Whitney U test was used to investigate if some features significantly change when the contrast of the echogenic changes. Our results showed a high differentiation performance of textural features across different echogenic objects. Cluster shade, correlation, MCC, and IMC2 of GLCM were the most discriminative features across the three echo objects of hypoechoic, isoechoic, and background. Cluster shade measures the skewness and uniformity of the GLCM. The higher the cluster shade, the greater the asymmetry from the mean. The correlation feature examines the linear relationship between the value of each gray level and its corresponding voxel in the GLCM matrix. The value of this feature varies between zero and one. The more linear the dependence, the closer the value of the correlation feature to one. MCC and IMC2 are measurements of the complexity of the texture of GLCM matrix. The values of the all four features were higher in the hypoechoic objects compare with isoechoic and background ones (fig. 2).

Our study had some limitations. The study was constructed based on a phantom ; In future studies, it should be better to increase the number of patients, Furthermore, since 2D B-mode ultrasound imaging was considered in this study, we only extracted textural radiomics features from 2D ROIs. Another limitation of the study was that we used only the textural features introduced by previous studies.

Conclusions

In summary, we concluded that in ultrasound imaging, even in different scan settings and parameters, several robust textural radiomics features may be found that are stable in various scan settings, as well as can differentiate various echogenic objects.

References:

[1]	P. Lambin et al., Radiomics: extracting more information from medical images using advanced feature analysis. European journal of cancer (Oxford, England : 1990) 48, 441-446 (2012).

[2]	V. Kumar et al., Radiomics: the process and the challenges. Magnetic resonance imaging 30, 1234-1248 (2012).

[3]	R. J. Gillies, P. E. Kinahan, H. Hricak, Radiomics: Images Are More than Pictures, They Are Data. Radiology 278, 563-577 (2016).

[4]	A. Chaddad, S. Sabri, T. Niazi, B. Abdulkarim, Prediction of survival with multi-scale radiomic analysis in glioblastoma patients. Medical & biological engineering & computing 56, 2287-2300 (2018).

[5]	L. Duron et al., Gray-level discretization impacts reproducible MRI radiomics texture features. PLoS One 14, e0213459 (2019).

[6]	K. Jang, C. Russo, A. Di Ieva, Radiomics in gliomas: clinical implications of computational modeling and fractal-based analysis. Neuroradiology 62, 771-790 (2020).

[7]	M. Avanzo et al., Machine and deep learning methods for radiomics. Medical physics 47, e185-e202 (2020).

[8]	S. S. Yip, H. J. Aerts, Applications and limitations of radiomics. Physics in medicine and biology 61, R150-166 (2016).

[9]	A. Traverso, L. Wee, A. Dekker, R. Gillies, Repeatability and Reproducibility of Radiomic Features: A Systematic Review. International Journal of Radiation Oncology, Biology, Physics 102, 1143-1158 (2018).

[10]	X. Guo et al., Deep learning radiomics of ultrasonography: Identifying the risk of axillary non-sentinel lymph node involvement in primary breast cancer. EBioMedicine 60, 103018 (2020).

[11]	Y. Sim, S. E. Lee, E.-K. Kim, S. Kim, A Radiomics Approach for the Classification of Fibroepithelial Lesions on Breast Ultrasonography. Ultrasound in Medicine & Biology 46, 1133-1141 (2020).

[12]	Y. Peng et al., Ultrasound-Based Radiomics Analysis for Preoperatively Predicting Different Histopathological Subtypes of Primary Liver Cancer. 10,  (2020).

[13]	X. Qiu et al., Could Ultrasound-Based Radiomics Noninvasively Predict Axillary Lymph Node Metastasis in Breast Cancer? 39, 1897-1905 (2020).

[14]	J. C. D’Souza et al., B-mode ultrasound for the assessment of hepatic fibrosis: a quantitative multiparametric analysis for a radiomics approach. Scientific Reports 9, 8708 (2019).

[15]	L. O. Osapoetra et al., Assessment of clinical radiosensitivity in patients with head-neck squamous cell carcinoma from pre-treatment quantitative ultrasound radiomics. Scientific Reports 11, 6117 (2021).

[16]	H. Zhou et al., Differential Diagnosis of Benign and Malignant Thyroid Nodules Using Deep Learning Radiomics of Thyroid Ultrasound Images. European Journal of Radiology 127, 108992 (2020).

[17]	B. Theek, T. Opacic, Z. Magnuska, T. Lammers, F. Kiessling, Radiomic analysis of contrast-enhanced ultrasound data. Scientific Reports 8, 11359 (2018).

[18]	H. Mi et al., Impact of different scanners and acquisition parameters on robustness of MR radiomics features based on women’s cervix. Scientific Reports 10, 20407 (2020).

[19]	J. Lee et al., Radiomics feature robustness as measured using an MRI phantom. Scientific Reports 11, 3973 (2021).

[20]	C. Haarburger et al., Radiomics feature reproducibility under inter-rater variability in segmentations of CT images. Scientific Reports 10, 12688 (2020).

[21]	E. Pfaehler et al., Repeatability of (18) F-FDG PET radiomic features: A phantom study to explore sensitivity to image reconstruction settings, noise, and delineation method. Med Phys 46, 665-678 (2019).

[22]	P. Lambin et al., Radiomics: the bridge between medical imaging and personalized medicine. Nature Reviews Clinical Oncology 14, 749-762 (2017).